8 channel Rail RS485 Relay commamd

1 This version has 2 Command modes, MODBUS RTU Command and AT Command.  2 The default Command is the MODBUS RTU Command, compatible with older versions.

3 Switch to AT command by shorting the M0 pad.

4 The default baud rate is 9600BPS. You can also select the baud rate by shorting the M1 and M2 pads.

| 9600BPS(default) | 2400BPS | 4800BPS | 19200BPS |
| --- | --- | --- | --- |

MODBUS Command (function code 06 is Control command,03 is Read status command)

Note :

1 MODBUS command must be HEX

2 Slave ID (device address) must be consistent with the DIP switches (A0-A5)

3 Jumper switch status: M0's two pads must be disconnected , as shown

9600 Band ,8 Data bits,None Parity,1 Stop Bit。

MODBUS 06 Command (Control command ,HEX):

| Bytes Number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MODBUS Definitions | Slave ID | Function | Address |  | Data |  | CRC Check |  |
| Function | Device Address | Function | Channel number |  | Command | Delay time | CRC Check |  |
| Open | 0x00-0x2F | 0x06 | 0x0001- 0x0008 |  | 0x01 | 0x00 | 2Bytes CRC |  |
| Close | 0x00-0x2F | 0x06 | 0x0001- 0x0008 |  | 0x02 | 0x00 | 2Bytes CRC |  |
| Toggle (Self-locking) | 0x00-0x2F | 0x06 | 0x0001- 0x0008 |  | 0x03 | 0x00 | 2Bytes CRC |  |
| Latch Inter-locking) | 0x00-0x2F | 0x06 | 0x0001- 0x0008 |  | 0x04 | 0x00 | 2Bytes CRC |  |
| Momentary (Non-locking) | 0x00-0x2F | 0x06 | 0x0001- 0x0008 |  | 0x05 | 0x00 | 2Bytes CRC |  |
| Delay | 0x00-0x2F | 0x06 | 0x0001- 0x0008 |  | 0x06 | 0x00-0xff | 2Bytes CRC |  |
| Open all | 0x00-0xFE | 0x06 | 0x0000 |  | 0x07 | 0x00 | 2Bytes CRC |  |
| Close all | 0x00-0xFE | 0x06 | 0x0000 |  | 0x08 | 0x00 | 2Bytes CRC |  |

Remarks:

1 Momentary mode, delay time is 1 seconds

2 Delay mode, delay time is 0-255 seconds

Return command：

Command is active, return to send com

mands; instruction is invalid no return.

MODBUS 03 Command (Read status command ,HEX):

| Bytes Number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MODBUS Definitions | Slave ID | Function | Address |  | Data |  | CRC Check |  |
| Function | Device Address | Function | Starting register address |  | Register length |  | CRC Check |  |
| Read Channel 1 State | 0x00-0x2F | 0x03 | 0x0001 |  | 0x0001 |  |  |  |
| Read Channel 2 State | 0x00-0x2F | 0x03 | 0x0002 |  | 0x0001 |  |  |  |
| Read 2 consecutive channels status | 0x00-0x2F | 0x03 | 0x0001-0x0003 |  | 0x0002 |  |  |  |
| Read 3 consecutive channels status | 0x00-0x2F | 0x03 | 0x0001-0x0002 |  | 0x0003 |  |  |  |
| Read all 8 channels status | 0x00-0x2F | 0x03 | 0x0001 |  | 0x0008 |  |  |  |

Read status command returns (function code 03, HEX format):

| Bytes length | 1 | 1 | 1 |  | 2 |
| --- | --- | --- | --- | --- | --- |
| MODBUS Definitions | Slave ID | Function | data length | data | CRC16 Check |
| Function | Device Address | Function | data length | Relay state 0x0001 open 0x0000 close | CRC16 Check |
| Channel 1 open | 0x00-0x1F | 0x03 | 0x02 | 0x0001 |  |
| Channel 1 close | 0x00-0x1F | 0x03 | 0x02 | 0x0000 |  |
| Channel 2 open | 0x00-0x1F | 0x03 | 0x02 | 0x0001 |  |
| Channel 2 close | 0x00-0x1F | 0x03 | 0x02 | 0x0000 |  |
| Channel 1 open Channel 2 open | 0x00-0x1F | 0x03 | 0x04 | 0x0001 0x0001 |  |
| Channel 1 open Channel 2 close | 0x00-0x1F | 0x03 | 0x04 | 0x0001 0x0000 |  |
| Channel 1 close Channel 2 open | 0x00-0x1F | 0x03 | 0x04 | 0x0000 0x0001 |  |
| Channel 1 close Channel 2 close | 0x00-0x1F | 0x03 | 0x04 | 0x0000 0x0000 |  |

MODBUS commands you can use "Modbus Poll" input, as shown below

（CRC check generated automatically）

You can also use HyperTerminal serial input, as shown below

（Manually add CRC check）

Examples (Slave ID is 1,DIP switch state)

Channel 1 Open ：01 06 00 01 01 00 D9 9A

Channel 1 Close ：01 06 00 01 02 00 D9 6A

Channel 1 Toggle：01 06 00 01 03 00 D8 FA

Channel 1 Latch：01 06 00 01 04 00 DA CA

Channel 1 Momentary: 01 06 00 01 05 00 DB 5A

Channel 1 Delay 10 seconds : 01 06 00 01 06 0A 5B AD

Channel 1 Delay 100 seconds: 01 06 00 01 06 64 DA 41

Channel 2 Open ：01 06 00 02 01 00 29 9A

Channel 2 Close ：01 06 00 02 02 00 29 6A

Channel 2 Toggle ：01 06 00 02 03 00 28 FA

Channel 2 Latch ：01 06 00 02 04 00 2A CA

Channel 2 Momentary : 01 06 00 02 05 00 2B 5A

Channel 2 Delay 10 seconds  : 01 06 00 02 06 0A AB AD

Channel 2 Delay 100 seconds : 01 06 00 02 06 64 2A 41

Open all：01 06 00 00 07 00 8B FA

Close all：01 06 00 00 08 00 8E 0A

Read state (assuming that the channel 1 is open, the channel 2 is close).

Read channel 1 state ：01 03 00 01 00 01 D5 CA

Return open：01 03 02 00 01 79 84

Read channel 2 state ：01 03 00 02 00 01 25 CA

Return close：01 03 02 00 00 B8 44

Read channel 1 and channel 2 state ：01 03 00 01 00 02 95 CB

Return channel open and channel 2 close ：01 03 04 00 01 00 00 AB F3

AT command (ASCII characters)

Note:

1 In the AT command mode slave ID is invalid

2 AT commands must be uppercase, lowercase invalid

3 Jumper switch status: M0's two pads are soldered together, as shown

9600 Band ,8 Data bits,None Parity,1 Stop Bit

Read Status:

Channel 1: AT+R1

Channel 2: AT+R2

Channel 3: AT+R3

Channel 4: AT+R4

Channel 5: AT+R5

Channel 6: AT+R6

Channel 7: AT+R7

Channel 8: AT+R8

Open :

Channel 1 : AT+O1

Channel 2 : AT+O2

Channel 3: AT+O3

Channel 4: AT+O4

Channel 5: AT+O5

Channel 6: AT+O6

Channel 7: AT+O7

Channel 8: AT+O8

Close:

Channel 1 : AT+C1

Channel 2 : AT+C2

Channel 3 : AT+C3

Channel 4 : AT+C4

Channel 5: AT+C5

Channel 6: AT+C6

Channel 7: AT+C7

Channel 8: AT+C8

Toggle (Self-locking)

Channel 1: AT+T1

Channel 2: AT+T2

Channel 3: AT+T3

Channel 4: AT+T4

Channel 5: AT+T5

Channel 6: AT+T6

Channel 7: AT+T7

Channel 8: AT+T8

Latch (Inter-locking)

Channel 1: AT+L1

Channel 2: AT+L2

Channel 3: AT+L3

Channel 4: AT+L4

Channel 5: AT+L5

Channel 6: AT+L6

Channel 7: AT+L7

Channel 8: AT+L8

Momentary (Non-locking)

Channel 1: AT+M1

Channel 2: AT+M2

Channel 3: AT+M3

Channel 4: AT+M4

Channel 5: AT+M5

Channel 6: AT+M6

Channel 7: AT+M7

Channel 8: AT+M8

Delay

Channel 1: AT+D1=XXXX

Channel 2: AT+D2=XXXX

Channel 3: AT+D3=XXXX

Channel 4: AT+D4=XXXX

Channel 5: AT+D5=XXXX

Channel 6: AT+D6=XXXX

Channel 7: AT+D7=XXXX

Channel 8: AT+D8=XXXX

XXXX refers to the 0000 to 9999 figures, Unit is seconds

All Relays Open

AT+AO

All Relays Close

AT+AC

Return command : OpenX, CloseX (X = 1/2/3/4/5/6/7/8)

Example 1:

Send command "AT+D1=0010", Channel 1 is "Open", after delay of 10 seconds, channel 1 is "Close"

Send command "AT+D2=0100", Channel 2 is "Open", after delay of 100 seconds, channel 2 is "Close"

Example 2:

Send command "AT+L1", Channel 1 is "Open", other Channels is "Close"

Send command "AT+L2", Channel 2 is "Open", other Channels is "Close"